FLOATING POINT ROM ENTRY POINTS

Entry  Addr Function

Point

AFP    D800 Extract a floating point

number from an ASCII str.

(FR0<--[INBUFF+CIX])

FASC   D8E6 Convert a floating point

number to an ASCII string

returns pointer in INBUFF

IFP    D9AA Convert a 2-byte integer

(FR0) to a floating point

number (in FR0)

FPI    D9D2 Convert a floating point

number to an integer

ZFR0   DA44 Zero the floating point

register (FR0)

ZF1    DA46 Zero 6 zero page bytes

pointed to by "X"

ZPG0   DA48 Zero "Y" bytes pointed to

by "X"

INITBF DA51 Store the address of

LBUFF in INBUFF

DBLZ2  DA5A Shift Z2 left one bit

(16-bit High/Low format)

FSUB   DA60 Subtract FR1 from FR0

FADD   DA66 Add FR1 to FR0

(result in FR0)

FMUL   DADB Multiply FR0 by FR1

(result in FR0)

FDIV   DB28 Divide FR0 by FR1

(result in FR0)

SKBLK  DBA1 Skip 0 or more blanks

pointed to by

[INBUFF,CIX], result

modifies CIX

GCHAR  DBAF Read a byte, convert it

as an ASCII decimal

number, set CY if it is

not

SH0L4  DBE4 Shift the contents of FR0

left 4 bits

NORMAL DC00 Normalize the contents of

FR0

PLYEVL DD40 Evaluate the polynomial

[Y.X](FR0), with A terms

FLD0R  DD89 Load FR0 from [Y.X]

FLD0P  DD8D Load FR0 from [FLPTR]

FLD1R  DD98 Load FR1 from [Y.X]

FLD1P  DD9C Load FR1 from [FLPTR]

FST0R  DDA7 Store FR0 into 6 bytes

pointed to by [Y.X]

FST0P  DDAB Store FR0 into [FLPTR]

FMOVE  DDB6 Copy FR0 over into FR1

EXP    DDC0 FR0 <-- exponential

function of FR0

EXP10  DDCC FR0 <-- 10^FR0

(exponent of 10)

REDARG DE95 FR0 <--

(FR0-[Y.X])/(FR0+[Y.X])

LOG    DECD FR0 <-- natural log of

FR0

LOG10  DED1 FR0 <-- base 10 log of

FR0

HALF   DF6C Constant = 0.5000000000

PLYSIN DFAE 11 Constant table for SIN

polynomial

NONE   DFEA Constant = 0.9999999999

SIN45  DFF0 Constant = sin(45 deg.)

Entry points not in the ATARI "D" ROM

RSTARG DD22 Copy PLYARG over into FR1

LDARG  DD2D Copy PLYARG over into FR0

PLYONE DD38 Same as PLYEVL except

implicit 1.0 coefficient

LDPLY  DD62 Copy [FPTR2] into FR1,

add 6 to FPTR2

SAVARG DD78 Store X into FPTR2, Y

into FPTR2+1, copy FR0

over into PLYARG